Mathematical Principles and Boundary Conditions

Perovskite Solar Cell Drift-Diffusion Simulator

(perovskite-sim)

March 2026

1. Introduction

perovskite-sim is a one-dimensional drift-diffusion solver with ion migration for modeling perovskite halide (MAPbI₃) solar cells. The simulator captures J-V hysteresis, impedance spectroscopy, and long-term degradation under applied bias through a fully coupled system of partial differential equations. The code operates on a non-uniform spatial grid and solves:

• Poisson's equation for electrostatic potential
• Electron and hole continuity equations via Scharfetter-Gummel discretization
• Ion vacancy continuity equation with steric (excluded-volume) correction

This technical reference document details the mathematical formulation, discretization schemes, boundary conditions, and material parameters used in the simulator.

2. Governing Equations

2.1 Poisson Equation (Electrostatics)

d/dx(ε₀εᵣ dφ/dx) = -ρ

where ρ = q(p − n + P − Nₐ + Nᴅ)

φ: electrostatic potential [V]

ρ: charge density [C/m³]

P: ion vacancy density [m⁻³]

Nₐ, Nᴅ: acceptor/donor doping densities

2.2 Electron Continuity Equation

∂n/∂t = (1/q) ∂Jₙ/∂x + G − R

Jₙ = qDₙ(∂n/∂x) + qμₙn(∂φ/∂x)

Using Scharfetter-Gummel discretization:

Jₙ = qDₙ/h [B(ξ)nᵢ₊₁ − B(−ξ)nᵢ]

where ξ = (φᵢ₊₁ − φᵢ)/Vₜ and B(x) = x/(eˣ − 1) is the Bernoulli function.

2.3 Hole Continuity Equation

∂p/∂t = −(1/q) ∂Jₚ/∂x + G − R

Jₚ = qDₚ/h [B(ξ)pᵢ − B(−ξ)pᵢ₊₁]

2.4 Ion Migration (Steric Blakemore Model)

∂P/∂t + ∂Fₚ/∂x = 0

Fₚ = −Dᵢ (∂P/∂x)/(1 − P_avg/P_lim) + (Dᵢ/Vₜ) P_avg ∂φ/∂x

The steric term 1/(1 − P/P_lim) enforces excluded-volume saturation, preventing unphysical ion densities exceeding the limiting ion concentration.

3. Recombination Mechanisms

3.1 Shockley-Read-Hall (SRH)

R_SRH = (np − nᵢ²) / [τₚ(n + n₁) + τₙ(p + p₁)]

where n₁ = Nᴄ exp(Eₜ/kT), p₁ = Nᵥ exp(−Eₜ/kT)

3.2 Radiative (Bimolecular)

R_rad = B_rad (np − nᵢ²)

3.3 Auger

R_Auger = (Cₙn + Cₚp)(np − nᵢ²)

Total recombination rate: R = R_SRH + R_rad + R_Auger

4. Optical Generation

Generation is computed using the Beer-Lambert law on each absorbing layer:

G(x) = Φ α(x) exp(−∫₀ˣ α(x′) dx′)

Φ: AM1.5G photon flux (~2.5×10²¹ m⁻² s⁻¹)

α: absorption coefficient [m⁻¹]

Cumulative optical depth computed via trapezoidal integration

5. Boundary Conditions

5.1 Electrical (Dirichlet for Poisson)

φ(0) = 0 (left contact)

φ(L) = V_bi − V_app (right contact)

Forward bias (V_app > 0) reduces the built-in electric field, enabling carrier collection across the absorber.

5.2 Ohmic Contacts (Carrier Densities)

n_contact = (Nᴅ − Nₐ)/2 + √[(Nᴅ − Nₐ)²/4 + nᵢ²]

p_contact = nᵢ²/n_contact

Two-branch formula avoids numerical overflow: n-type branch for Nᴅ ≥ Nₐ, p-type branch otherwise.

5.3 Ion Zero-Flux

∂P/∂x = 0 at both contacts (ions confined to bulk)

Implemented via zero-flux padding: F[0] = F[N] = 0

6. Spatial Discretization

6.1 Tanh-Clustered Grid

ξ = tanh(α ξ̃) / tanh(α),  ξ̃ ∈ [−1, 1]

x(ξ) = L (1 + ξ) / 2

α = 3.0 concentrates grid points near domain boundaries (junctions), where electric field gradients are steepest.

6.2 Finite-Volume Operators

Dual-grid cell widths: h_cell_i = 0.5(hᵢ₋₁ + hᵢ) for internal nodes

Harmonic-mean face permittivity: εₓ = 2εₐεᵦ/(εₐ + εᵦ)

Harmonic-mean diffusion coefficients: D_face = 2DₐDᵦ/(Dₐ + Dᵦ)

Both correspond to a series-resistance model for sharp material discontinuities.

6.3 Bernoulli Function (Numerical Stability)

B(x) = x / (eˣ − 1)

For |x| < 10⁻⁸: B(x) ≈ 1 − x/2 + x²/12 (Taylor series)

For |x| ≥ 10⁻⁸: B(x) = x / expm1(x) (numerically stable)

7. Time Integration

7.1 Method of Lines (MOL)

State vector y = [n, p, P] (3N components) solved by Radau IIA (5th order implicit Runge-Kutta).

Radau IIA advantages:

Handles stiffness from exponential Boltzmann factors

5th-order accuracy

Default tolerances: rtol = 10⁻⁴, atol = 10⁻⁶

7.2 Operator Splitting (Fallback)

When Radau stalls (e.g., ion pile-up at interfaces):

Ion advance: freeze n, p; advance P alone

Carrier re-equilibration: run short transient (~1 μs) with new ion field

8. Physical Constants

| Constant | Symbol | Value (SI Units) |
| --- | --- | --- |
| Elementary charge | q | 1.602176634 × 10⁻¹⁹ C |
| Boltzmann constant | kᵦ | 1.380649 × 10⁻²³ J/K |
| Temperature | T | 300 K |
| Thermal voltage | Vₜ = kᵦT/q | ≈ 0.025852 V |
| Vacuum permittivity | ε₀ | 8.854187817 × 10⁻¹² F/m |

9. Material Parameters (MAPbI₃)

| Parameter | Symbol | Value |
| --- | --- | --- |
| Relative permittivity | εᵣ | 24.1 |
| Electron mobility | μₙ | 2 × 10⁻⁴ m²/V/s |
| Hole mobility | μₚ | 2 × 10⁻⁴ m²/V/s |
| Electron diffusivity | Dₙ | 5.17 × 10⁻⁶ m²/s |
| Hole diffusivity | Dₚ | 5.17 × 10⁻⁶ m²/s |
| Intrinsic carrier density | nᵢ | 3.2 × 10¹³ m⁻³ |
| Electron lifetime | τₙ | 1 × 10⁻⁶ s |
| Hole lifetime | τₚ | 1 × 10⁻⁶ s |
| Radiative recombination | Bᵣₐₐ | 5 × 10⁻²² m³/s |
| Electron Auger coeff. | Cₙ | ≈ 1 × 10⁻⁴² m⁶/s (negligible) |
| Hole Auger coeff. | Cₚ | ≈ 1 × 10⁻⁴² m⁶/s (negligible) |
| Absorption coefficient | α | 1.3 × 10⁷ m⁻¹ |

Ion Migration Parameters:

| Parameter | Symbol | Value |
| --- | --- | --- |
| Ion diffusivity | Dᵢ | 1 × 10⁻¹⁶ m²/s |
| Ion saturation density | P_lim | 1.6 × 10²⁷ m⁻³ |
| Initial ion vacancy density | P₀ | 1.6 × 10²⁴ m⁻³ |

10. Correctness Notes and Known Limitations

10.1 Duplicate Q Constant

The elementary charge constant Q is redefined locally in continuity.py, fe_operators.py, and newton.py instead of being imported from constants.py. While all local definitions use identical values, this represents a maintenance risk: future changes to the reference value in constants.py will not automatically propagate to these modules.

10.2 Beer-Lambert Illumination Direction

The code assumes light always enters from x = 0 (left contact). For n-i-p architectures where light physically enters through the electron-transport layer (right side), the simulated generation profile is reversed relative to the physical setup. Given α ≈ 1.3 × 10⁷ m⁻¹ (1/e characteristic depth ~77 nm), this directional difference significantly affects the spatial distribution of photogenerated carriers and thus impacts J-V characteristics.

10.3 Uniform Ion Diffusivity

The assemble_rhs function applies absorber.params.D_ion uniformly across the entire device grid, rather than using per-layer or per-node diffusivity values. While initial ion density is zero in transport layers, this permits spurious ion diffusion into those regions during simulation, which may not reflect physical reality where ions are largely confined to the halide perovskite absorber.

10.4 Degradation PCE Proxy

Power conversion efficiency (PCE) computed for degradation studies uses open-circuit voltage (Vₒc) derived from the geometric mean of quasi-Fermi level separation, which overestimates by approximately 100–150 mV relative to rigorous Boltzmann-transport calculation. Additionally, the PCE metric lacks fill factor integration, making it a proxy metric rather than a complete performance indicator.